CLEANWELL SAFETY DATA SHEET

AUTO DISHWASH DETERGENT

Issued: 01/08/2008










Revision No: 3
1. IDENTIFICATION OF THE PREPARATION AND SUPPLIER
Product Name:

AUTO DISHWASH DETERGENT



Product code: 

303









Use / description of product: 303: A concentrated low foaming dishwashing liquid. Effective with hard or soft 


water, in automatic and conveyor machines.
Supplier:

CLEANWELL, 
Smithstown Ind. Estate, 
Shannon, Co. Clare.

Telephone: 
061 360191 
Facsimile: 
061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
• /-TRISODIUM NITRILOTRIACETATE 10-30% 

EINECS: 225-768-6 CAS: 5064-31-3 

[Xn] R22; [Xi] R36

•SODIUM HYDROXIDE 10-30% 
EINECS: 215-185-5 CAS: 1310-73-2 
[C] R35

3. HAZARDS IDENTIFICATION  
Main hazards: 

Causes severe burns.

4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 
Blistering may occur. Progressive ulceration will occur if treatment is not immediate.

Eye contact: 

Corneal burns may occur. May cause permanent damage.

Ingestion: 
Corrosive burns may appear around the lips. Blood may be vomited. There may be bleeding from the mouth or nose.

Inhalation: 
There may be shortness of breath with a burning sensation in the throat. Exposure may cause coughing or wheezing.

4. FIRST AID MEASURES (ACTION)

Skin contact: 
Remove all contaminated clothes and footwear immediately unless stuck to skin. Drench the affected skin with running water for 10 minutes or longer if substance is still on skin. Transfer to hospital if there are burns or symptoms of poisoning.

Eye contact: 
Bathe the eye with running water for 15 minutes. Transfer to hospital for specialist examination.

Ingestion: 

Wash out mouth with water. Do not induce vomiting. Give 1 cup of water to drink 


every 10 minutes. If unconscious, check for breathing and apply artificial respiration 


if necessary. If unconscious and breathing is OK, place in the recovery position. 



Transfer to hospital as soon as possible.

Inhalation: 

Remove casualty from exposure ensuring one's own safety whilst doing so. If 



unconscious and breathing is OK, place in the recovery position. If conscious, 



ensure the casualty sits or lies down. If breathing becomes bubbly, have the casualty 


sit and provide oxygen if available. Transfer to hospital as soon as possible.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Suitable extinguishing media for the surrounding fire should be used. Use water 



spray to cool containers.

Exposure hazards: 
Corrosive. In combustion emits toxic fumes.

Protection of fire fighters: Wear self-contained breathing apparatus. Wear protective clothing to prevent 



contact with skin and eyes.

6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Notify the police and fire brigade immediately. If outside keep bystanders upwind 


and away from danger point. Mark out the contaminated area with signs and prevent 


access to unauthorised personnel. Do not attempt to take action without suitable 



protective clothing - see section 8 of SDS. Do not create dust.

Environmental precautions: Do not discharge into drains or rivers.

Clean-up procedures: 
Clean up should be dealt with only by qualified personnel familiar with the specific 


substance.  Transfer to a closable, labeled salvage container for disposal by an 



appropriate method.

7. HANDLING AND STORAGE

Handling requirements: Avoid direct contact with the substance. Ensure there is sufficient ventilation of the 


area. Do not handle in a confined space. Avoid the formation or spread of dust in the 


air.

Storage conditions: 
Store in cool, well-ventilated area. Keep container tightly closed.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Hazardous ingredients: 
SODIUM HYDROXIDE

WEL (15 min STEL): 2 mg/m3
Hand protection: 
Protective gloves.

Eye protection: 

Tightly fitting safety goggles. Ensure eye bath is to hand.

Skin protection: 

Protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid



Colour: 


Yellow-orange

Odour: 


Barely perceptible odour

Evaporation rate: 
Moderate

Solubility in water: 
Soluble



Viscosity: 

Non-viscous

Relative density: 
1.05 - 1.15


PH: 


1% = 12 - 12.5

VOC g/l:


0
10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Conditions to avoid: 
Heat.

Materials to avoid: 
Strong oxidising agents. Strong acids.

Haz. decomp. products: 
In combustion emits toxic fumes.

11. TOXICOLOGICAL INFORMATION

Hazardous ingredients: 
SODIUM HYDROXIDE



IPR MUS LD50 40 mg/kg




ORL RBT LDLO 500 mg/kg

Routes of exposure: 
Refer to section 4 of SDS for routes of exposure and corresponding symptoms.

12. ECOLOGICAL INFORMATION

Persistence and degradability: Biodegradable.

Bio-accumulative potential: No bioaccumulation potential.

Other adverse effects: 
Negligible eco-toxicity.

13. DISPOSAL CONSIDERATIONS

Disposal of packaging: 
Dispose of as normal industrial waste.

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.
14. TRANSPORT INFORMATION

ADR / RID

UN no: 

1760


ADR Class:

 8

Packing group: III 


Classification code: 
C9
Shipping name: CORROSIVE LIQUID, N.O.S.
 (SODIUM HYDROXIDE (12%))
Labelling: 
8 


Hazard ID no: 

80

[image: image1.emf]
IMDG / IMO

UN no: 

1760


Class:

 8

Packing group: III 


EmS: 

F-A,S-B

Marine pollutant: 


Labeling: 
8

IATA / ICAO

UN no: 

1760


Class: 

8

Packing group: 
III 
Packing instructions: 809(P&CA); 812(CAO)
Labeling: 8

15. REGULATORY INFORMATION

Hazard symbols:
Corrosive.
[image: image2.png]




Risk phrases: 

R35: Causes severe burns.

Safety phrases: 
S26: In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. 

S36/37/39: Wear suitable protective clothing, gloves and eye / face protection.

S45: In case of accident or if you feel unwell, seek medical advice immediately 
(show the label where possible).

Seveso II guideline: 
No

Water hazard class: 
1 Classification by VwVwS

Note: 
The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions that complete these regulations. Refer to all applicable national, international and local regulations or provisions.

16. OTHER INFORMATION

Risk phrases used in s.2: 
R22: Harmful if swallowed.




R36: Irritating to eyes.




R35: Causes severe burns.

Legal disclaimer: The above information is believed to be correct but does not purport to be all-inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.
- 3 -

