CLEANWELL SAFETY DATA SHEET

CLEANER DEGREASER


Issued: 01/10/2008










Revision No: 1
1. IDENTIFICATION OF THE / PREPARATION AND SUPPLIER
Product name: 

CLEANER DEGREASER
Product code: 

301

Use / description of product: 301; A concentrated multi purpose cleaner for use on textured, porous and 



smooth alkali resistant surfaces

Company name: 
CLEANWELL

                             Smithstown Ind. Estate,

              

               Shannon, Co. Clare

                             
Telephone:            061 360191


                             Fax:                       061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
C10 -C16 PARETH (2 - 10 ) 1-10%

CAS: 127036-24-2

[Xi] R41

• (C9 - C18)PARETH(2 - 18) 1-10%

• 3 - BUTOXYPROPAN - 2 - OL 1-10%

EINECS: 225-878-4 CAS: 005131-66-8

[Xi] R36/38

• (C12 - C18)PARETH(2 -18) 1-10%

CAS: 24938-91-8

[Xn] R22; [Xi] R41

• SODIUM HYDROXIDE <1%

EINECS: 215-185-5 CAS: 1310-73-2

[C] R35

• AMPHOTERIC SURFACTANT 1-10%

EINECS :)

[Xi] R36

3. HAZARDS IDENTIFICATION

Main hazards: 

Irritating to eyes and skin.

4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 

There may be irritation and redness at the site of contact.

Eye contact: 

There may be pain and redness. The eyes may water profusely. There may be 



severe pain. The vision may become blurred. May cause permanent damage.

Ingestion: 

There may be soreness and redness of the mouth and throat. Nausea and 



stomach pain may occur.

Inhalation: 

There may be irritation of the throat with a feeling of tightness in the chest.

4. FIRST AID MEASURES (ACTION)

Skin contact: 

Remove all contaminated clothes and footwear immediately unless stuck to skin. 


Wash immediately with plenty of soap and water.

Eye contact: 

Bathe the eye with running water for 15 minutes. Transfer to hospital for 



specialist examination.

Ingestion: 

Wash out mouth with water. Do not induce vomiting. If conscious, give half a 



litre of water to drink immediately. Consult a doctor.

Inhalation: 

Remove casualty from exposure ensuring one's own safety whilst doing so.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Suitable extinguishing media for the surrounding fire should be used. Use water 



spray to cool containers.
Exposure hazards: 
In combustion emits toxic fumes.

Protection of fire-fighters: Wear self-contained breathing apparatus. Wear protective clothing to prevent 



contact with skin and eyes.

6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Mark out the contaminated area with signs and prevent access to unauthorised 



personnel. Do not attempt to take action without suitable protective clothing - 



see section 8 of SDS. Turn leaking containers leak-side up to prevent the escape 



of liquid.

Environmental precautions: Do not discharge into drains or rivers. Contain the spillage using bunding.

Clean-up procedures: 
Absorb into dry earth or sand. Transfer to a closable, labeled salvage container 



for disposal by an appropriate method.

7. HANDLING AND STORAGE

Handling requirements: 
Avoid direct contact with the substance. Ensure there is sufficient ventilation of 



the area. Avoid the formation or spread of mists in the air.

Storage conditions: 
Store in cool, well ventilated area. Keep container tightly closed.
Suitable packaging:
Stainless steel. Teflon. Glass.
8. EXPOSURE CONTROLS / PERSONAL PROTECTION
Hazardous ingredients: 
SODIUM HYDROXIDE

WEL (15 min STEL): 2 mg/m3

Engineering measures: 
Ensure there is sufficient ventilation of the area.

Hand protection: 
Protective gloves.  Gloves (alkali-resistant).  Viton gloves.  PVC gloves.
Eye protection: 

Tightly fitting safety goggles. Ensure eye bath is to hand.

Skin protection: 

Protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid



Colour: 


Blue

Odour: 


Characteristic odour

Evaporation rate: 
Moderate

Oxidising: 

Non-oxidising (by EC criteria)
Solubility in water: 
Soluble

Viscosity: 

Non-viscous


Relative density: 
1.05 - 1.15

PH: 


12.75 - 13.25


VOC g/l:

57.3
10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Materials to avoid: 
Strong oxidising agents. Strong acids.

Haz. decomp. products: 
In combustion emits toxic fumes.

11. TOXICOLOGICAL INFORMATION
Hazardous ingredients: 
SODIUM HYDROXIDE

IPR MUS LD50 40 mg/kg

ORL RBT LDLO 500 mg/kg

Routes of exposure: 
Refer to section 4 of SDS for routes of exposure and corresponding symptoms.

12. ECOLOGICAL INFORMATION

Mobility: 

Readily absorbed into soil.

Persistence and degradability: Biodegradable.

Bio-accumulative potential: No bioaccumulation potential.

Other adverse effects: 
Negligible eco-toxicity.

13. DISPOSAL CONSIDERATIONS

Disposal of packaging: 
Dispose of as normal industrial waste.

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.

14. TRANSPORT INFORMATION

ADR / RID

UN no: - NOT CLASSIFIED


IMDG / IMO

UN no: - NOT CLASSIFIED


IATA / ICAO

UN no: - NOT CLASSIFIED
15. REGULATORY INFORMATION

Hazard symbols: 
Irritant.
[image: image1.emf]
Risk phrases: 

R36/38: Irritating to eyes and skin.

Safety phrases: 

S2: Keep out of the reach of children.




S26: In case of contact with eyes, rinse immediately with plenty of water and 



seek medical advice.
Haz. ingredients (label): 
SODIUM HYDROXIDE
Seveso II guideline: 
No
Water hazard class: 
1 Classification by VwVwS
Air emission directive: 
Amount m%: 5.73
Note: The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions which complete these regulations. Refer to all applicable national, international and local regulations or provisions.
16. OTHER INFORMATION

Risk phrases used in s.2: 
R36/38: Irritating to eyes and skin.




R22: Harmful if swallowed.




R41: Risk of serious damage to eyes.




R35: Causes severe burns.




R36: Irritating to eyes.

Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.

