CLEANWELL SAFETY DATA SHEET

DESCALER

Issued: 21/08/2008
 







Revision No: 2
1. IDENTIFICATION OF THE PREPARATION AND SUPPLIER
Product name:

DESCALER
Product code: 

508

Use / description of product: 508: A super concentrated food, plant and equipment descaler. Descaler is a 



high purity, nonfoaming, odourless product.

Company name: 
CLEANWELL
                              
Smithstown Ind. Estate,

                              
Shannon, Co. Clare

                              
Telephone:            061 360191

                              
Fax:                       061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
ORTHOPHOSPHORIC ACID 10-30%




EINECS: 231-633-2 CAS: 7664-38-2




[C] R34

3. HAZARDS IDENTIFICATION

Main hazards: 

Causes burns.

4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 

Blistering may occur. Progressive ulceration will occur if treatment is not 



immediate.

Eye contact: 

Corneal burns may occur. May cause permanent damage.

Ingestion: 

Corrosive burns may appear around the lips. Blood may be vomited. There may 



be bleeding from the mouth or nose.

Inhalation: 

There may be shortness of breath with a burning sensation in the throat. 



Exposure may cause coughing or wheezing.

4. FIRST AID MEASURES (ACTION)

Skin contact: 

Remove all contaminated clothes and footwear immediately unless stuck to skin. 


Drench the affected skin with running water for 10 minutes or longer if 




substance is still on skin. Transfer to hospital if there are burns or symptoms of 



poisoning.

Eye contact: 

Bathe the eye with running water for 15 minutes. Transfer to hospital for 



specialist examination.

Ingestion: 

Wash out mouth with water. Do not induce vomiting. Give 1 cup of water to 



drink every 10 minutes. If unconscious, check for breathing and apply artificial 



respiration if necessary. If unconscious and breathing is OK, place in the 



recovery position. Transfer to hospital as soon as possible.

Inhalation: 

Remove casualty from exposure ensuring one's own safety whilst doing so. If 



unconscious and breathing is OK, place in the recovery position. If conscious, 



ensure the casualty sits or lies down. If breathing becomes bubbly, have the 



casualty sit and provide oxygen if available. Transfer to hospital as soon as 



possible.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Suitable extinguishing media for the surrounding fire should be used. Use water 



spray to cool containers.

Exposure hazards: 
Corrosive. In combustion emits toxic fumes.

Protection of fire-fighters: Wear self-contained breathing apparatus. Wear protective clothing to prevent 



contact with skin and eyes.
6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Notify the police and fire brigade immediately. If outside keep bystanders 



upwind and away from danger point. Mark out the contaminated area with signs 



and prevent access to unauthorised personnel. Do not attempt to take action 



without suitable protective clothing - see section 8 of SDS. Turn leaking 



containers leak-side up to prevent the escape of liquid.

Environmental precautions: Do not discharge into drains or rivers. Contain the spillage using bunding.

Clean-up procedures: 
Clean-up should be dealt with only by qualified personnel familiar with the 



specific substance. Absorb into dry earth or sand. Transfer to a closable, labeled 



salvage container for disposal by an appropriate method.

7. HANDLING AND STORAGE

Handling requirements: 
Avoid direct contact with the substance. Ensure there is sufficient ventilation of 



the area. Do not handle in a confined space. Avoid the formation or spread of 



mists in the air.

Storage conditions: 
Store in cool, well ventilated area. Keep container tightly closed.

Suitable packaging: 
Stainless steel. Polyethylene. Teflon.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Hazardous ingredients: 
ORTHOPHOSPHORIC ACID...100%




WEL (8 hr TWA): 1 mg/m3 WEL (15 min STEL): 2 mg/m3
Engineering measures: 
Ensure there is sufficient ventilation of the area.

Respiratory protection: 
Self-contained breathing apparatus must be available in case of emergency.

Hand protection: 
Impermeable gloves.

Eye protection: 

Tightly fitting safety goggles. Ensure eye bath is to hand.

Skin protection: 

Impermeable protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid 



Colour: 


Colourless

Odour: 


Barely perceptible odour 

Evaporation rate: 
Slow

Oxidising: 

Non-oxidising (by EC criteria) 
Solubility in water: 
Soluble

Viscosity: 

Non-viscous 


Relative density: 
1.15 - 1.25

pH: 


1% = 1.5 - 2.1

10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Conditions to avoid: 
Heat.

Materials to avoid: 
Strong oxidising agents. Strong acids.

Haz. decomp. products: 
In combustion emits toxic fumes.

11. TOXICOLOGICAL INFORMATION

Hazardous ingredients: 
ORTHOPHOSPHORIC ACID...100%




ORL RAT LD50 1530 mg/kg

Routes of exposure: 
Refer to section 4 of SDS for routes of exposure and corresponding symptoms.

12. ECOLOGICAL INFORMATION

Mobility: 

Readily absorbed into soil.

Persistence and degradability: Biodegradable.

Bio-accumulative potential: No bioaccumulation potential.

Other adverse effects: 
Negligible eco-toxicity.

13. DISPOSAL CONSIDERATIONS

Disposal of packaging: 
Dispose of as normal industrial waste.

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.

14. TRANSPORT INFORMATION

ADR / RID

UN no: 

3264 

ADR Class: 

8




Packing group: 
II 

Classification code: 
C1




Shipping name: 
CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. 





(ORTHOPHOSPHORIC ACID.28%)




Labeling: 
8 

Hazard ID no: 

80
[image: image1.emf]
IMDG / IMO

UN no: 

3264 

Class: 


8




Packing group: 
II 

EmS: 


F-A,S-B




Marine pollutant:

Labeling: 

8
IATA / ICAO 

UN no: 

3264 

Class: 


8




Packing group: 
II 

Labeling: 

8




Packing instructions: 818(P&CA); 820(CAO)
15. REGULATORY INFORMATION

Hazard symbols: 
Corrosive.



[image: image2.emf]


Risk phrases: 

R34: Causes burns.

Safety phrases: 

S26: In case of contact with eyes, rinse immediately with plenty of water and 



seek medical advice.




S36/37/39: Wear suitable protective clothing, gloves and eye / face protection.




S45: In case of accident or if you feel unwell, seek medical advice immediately 



(show the label where possible).

Seveso II guideline: 
No

Water hazard class: 
1 Classification by VwVwS
Note: The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions which complete these regulations. Refer to all applicable national, international and local regulations or provisions.

16. OTHER INFORMATION

Risk phrases used in s.2: 
R34: Causes burns.

Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.

