CLEANWELL SAFETY DATA SHEET

EXTRACTION CLEANER


Issued: 22/07/2010










Revision No: 4
1. IDENTIFICATION OF THE PREPARATION AND SUPPLIERS
Product name: 


EXTRACTION CLEANER
Product code: 


306
Use / description of product: 306: A fragrant low foam detergent for use on water washable colour fast 



fabrics, through spray extraction equipment.

Company name: 
CLEANWELL
                              
Smithstown Ind. Estate




Shannon, Co. Clare


                             Telephone:            061 360191

                              
Fax:                       061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
ALKYLAMINE CARBOXYLATE 1-2%



[Xi] R36

· C 12/15 A,  3-12 EO MULTIBRANCHED (DID 27) 1-2%

EINECS: POLYMER 
CAS: 24938-91-8

[Xn] R22;
[Xi] R41

· C9/11 A, >3-6 EO PREDOMINANTLY LINEAR (DID 21) 1-2%

EINECS: POLYMER
CAS: 160875-66-1

[Xi] R41

· SODIUM HYDROXIDE <1%

EINECS: 215-185-5
CAS: 1310-73-2

[C] R35
3. HAZARDS IDENTIFICATION

Main hazards: 

Irritating to eyes and skin.
4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 

There may be irritation and redness at the site of contact.

Eye contact: 

There may be pain and redness. The eyes may water profusely. There may be 



severe pain. The vision may become blurred. May cause permanent damage.

Ingestion: 

There may be soreness and redness of the mouth and throat. Nausea and 



stomach pain may occur.

Inhalation: 

There may be irritation of the throat with a feeling of tightness in the chest.

4. FIRST AID MEASURES (ACTION)

Skin contact: 
Wash immediately with plenty of soap and water. Transfer to hospital if there are burns or symptoms of poisoning.

Eye contact: 
Bathe the eye with running water for 15 minutes. Transfer to hospital for specialist examination.

Ingestion: 

Wash out mouth with water. Do not induce vomiting. If conscious, give half a 



litre of water to drink immediately. Transfer to hospital as soon as possible.

Inhalation: 

Remove casualty from exposure ensuring one's own safety whilst doing so. If 



conscious, ensure the casualty sits or lies down. If unconscious and breathing is 



OK, place in the recovery position. Transfer to hospital as soon as possible.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Suitable extinguishing media for the surrounding fire should be used. Use water 



spray to cool containers.

Exposure hazards: 
In combustion emits toxic fumes of carbon dioxide/carbon monoxide.
Protection of fire-fighters: Wear self-contained breathing apparatus. Wear protective clothing to prevent 



contact with skin and eyes.
6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Mark out the contaminated area with signs and prevent access to unauthorised 



personnel. Do not attempt to take action without suitable protective clothing - 



see section 8 of SDS. Turn leaking containers leak-side up to prevent the escape 



of liquid.

Environmental precautions: Do not discharge into drains or rivers. Contain the spillage using bunding.

Clean-up procedures: 
Absorb into dry earth or sand. Transfer to a closable, labelled salvage container 



for disposal by an appropriate method.

7. HANDLING AND STORAGE

Handling requirements: 
Avoid direct contact with the substance. Ensure there is sufficient ventilation of 



the area. Avoid the formation or spread of mists in the air.

Storage conditions: 
Store in cool, well ventilated area. Keep container tightly closed.

Suitable packaging: 
Stainless steel. Teflon. Polyethylene.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION
Hazardous ingredients:
SODIUM HYDROXIDE




WEL (15 min STEL): 2mg/m3

Engineering measures: 
Ensure there is sufficient ventilation of the area.

Respiratory protection: 
Respiratory protection not required.

Hand protection: 
Protective gloves. Gloves (alkali-resistant).

Eye protection: 

Tightly fitting safety goggles. Ensure eye bath is to hand.

Skin protection: 

Protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid



Colour: 


Green-blue

Odour: 


Pleasant



Evaporation rate: 
Moderate

Oxidising: 

Non-oxidising (by EC criteria)
Solubility in water: 
Soluble

Viscosity: 

Non-viscous


Boiling point/range oC:
100

Melting point/range oC:
0



Relative density: 
1.033 - 1.039

pH: 


1% = 10.5 - 11.5


VOC g/l:

0
10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Conditions to avoid: 
Heat.

Materials to avoid: 
Strong oxidising agents. Strong acids. Aluminium. Magnesium. Zinc.

Haz. decomp. products: 
In combustion emits toxic fumes of carbon dioxide/carbon monoxide.
11. TOXICOLOGICAL INFORMATION
Hazardous ingredients:
C12/15 A, 3-12 EO MULTIBRANCHED (DID 27)




ORL RAT LD50 1.5 g/kg

· C9/11 A, 5-11 EO MULTIBRANCHED (DID 23)

ORL RAT LD50 1400 mg/kg

· C9/11 A, >3-6 EO PREDOMINANTLY LINEAR (DID 21)
ORL RAT LD50 > 2000 mg/kg

· SODIUM HYDROXIDE

IPR MUS LD50 40 mg/kg

ORL RBT LDLO 500 mg/kg

Routes of exposure: 
Refer to section 4 of SDS for routes of exposure and corresponding symptoms.

12. ECOLOGICAL INFORMATION
Hazardous ingredients:
C 12/15 A, 3-12 EO MULTIBRANCHED (DID 27)




FISH 96H LC50 >1 mg/l

· C9/11 A, 5-11 EO MULTIBRANCHED (DID 23)

FISH 96H LC50 > 11.8 mg/l

· C9/11 a, > 3-6 EO PREDOMINANTLY LINEAR (DID 21)

ALGAE Scenedesmus 48H EC50 10 – 100 mg/l

Mobility: 

Readily absorbed into soil.

Persistence and degradability: Biodegradable. The surfactants contained in this preparation comply with 



The biodegradability criteria as laid down in regulation (EC) No 648/2004




on detergents.
Bioaccumulative potential: No bioaccumulation potential.

Other adverse effects: 
Negligible ecotoxicity.

13. DISPOSAL CONSIDERATIONS

Disposal operations: 
D1 Tipping above or underground (e.g. landfill, etc.).

Disposal of packaging: 
Dispose of as normal industrial waste.

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.
14. TRANSPORT INFORMATION

ADR / RID

UN no: - NOT CLASSIFIED


IMDG / IMO

UN no: - NOT CLASSIFIED
IATA / ICAO

UN no: - NOT CLASSIFIED
15. REGULATORY INFORMATION

Hazard symbols: 
Irritant.



[image: image1.emf]
Risk phrases: 

R36: Irritating to eyes.

Safety phrases: 

S2: Keep out of the reach of children.



S25: Avoid contact with eyes.



S26: In case of contact with eyes, rinse immediately with plenty of water and 



seek medical advice.
Haz. Ingredients (label):
SODIUM HYDROXIDE
Seveso II guideline: 
No

Water hazard class: 
1 Classification by VwVwS
Air emission directive:
Amount m%: 0
Note: The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions which complete these regulations. Refer to all applicable national, international and local regulations or provisions.

16. OTHER INFORMATION
Other information:
This safety data sheet is prepared in accordance with Regulation (EC) 
No 1907/2006.

Risk phrases used in s.2: 
R36: Irritating to eyes.




R22: Harmful if swallowed.

R41: Risk of serious damage to eyes.

R35: Causes severe burns.
Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.

