CLEANWELL SAFETY DATA SHEET

MAINTAINER 3


Issued: 17/04/2009







Revision No: 4
1. IDENTIFICATION OF THE PREPARATION AND THE COMPANY
Product name: 

MAINTAINER 3
Product code: 

104

Use / description of product: 104: Regular mopping or spray cleaning with MAINTAINER 3 creates a 



gloss finish and fragrant atmosphere.

Company name: 
CLEANWELL
                              
Smithstown Ind. Estate,


                              Shannon, Co. Clare


                              Telephone:            061 360191


                              Fax:                        061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
(C9-C18) PARETH (2-18). 1-10%

3. HAZARDS IDENTIFICATION

Main hazards: 

No significant hazard.

4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 

There may be mild irritation at the site of contact.

Eye contact: 

There may be irritation and redness.

Ingestion: 

There may be irritation of the throat.

Inhalation: 

No symptoms.

4. FIRST AID MEASURES (ACTION)

Skin contact: 

Wash immediately with plenty of soap and water.

Eye contact: 

Bathe the eye with running water for 15 minutes. Transfer to hospital for 



specialist examination.

Ingestion: 

Do not induce vomiting. Wash out mouth with water. If conscious, give half a 



litre of water to drink immediately. Transfer to hospital as soon as possible.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Suitable extinguishing media for the surrounding fire should be used. Use water 



spray to cool containers.

Exposure hazards: 
In combustion emits toxic fumes.

Protection of fire-fighters: Wear self-contained breathing apparatus. Wear protective clothing to prevent 



contact with skin and eyes.

6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Refer to section 8 of SDS for personal protection details. Turn leaking 




containers leak-side up to prevent the escape of liquid.

Environmental precautions: Do not discharge into drains or rivers. Contain the spillage using bunding.

Clean-up procedures: 
Absorb into dry earth or sand. Transfer to a closable, labelled salvage container 



for disposal by an appropriate method.

7. HANDLING AND STORAGE

Storage conditions: 
Store in cool, well ventilated area. Keep container tightly closed.
Suitable packaging:
Polythylene. Stainless steel.
8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Respiratory protection: 
Respiratory protection not required.

Hand protection: 
Protective gloves.

Eye protection: 

Safety glasses. Ensure eye bath is to hand.

Skin protection: 

Protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid

Colour: 


Pink

Odour: 


Sweet-smelling

Evaporation rate: 
Moderate

Oxidising: 

Non-oxidising (by EC criteria)

Solubility in water: 
Miscible

Viscosity: 

Non-viscous

Relative density: 
1 - 1.1

PH: 


7.5 – 9
VOC g/l:

20
10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Conditions to avoid: 
Heat.

Materials to avoid: 
Strong oxidising agents. 
Haz. decomp. products: 
In combustion emits toxic fumes.

11. TOXICOLOGICAL INFORMATION

Routes of exposure: 
Refer to section 4 of SDS for routes of exposure and corresponding symptoms.

12. ECOLOGICAL INFORMATION

Hazardous ingredients: 
(C9-C18) PARETH (2-18).



DAPHNIA 96H LC50 1 mg/l

Mobility: 

Readily absorbed into soil.

Persistence and degradability: Biodegradable. The surfactants contained in this preparation comply with 



the biodegradability criteria as laid down in regulation (EC) No 648/2004 on




detergents.
Bioaccumulative potential: No bioaccumulation potential.

Other adverse effects: 
Negligible ecotoxicity.

13. DISPOSAL CONSIDERATIONS
Disposal of packaging:
Dispose of as normal industrial waste.
NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.

14. TRANSPORT INFORMATION

ADR / RID 

UN no: - NOT CLASSIFIED


IMDG / IMO 

UN no: - NOT CLASSIFIED
IATA / ICAO 

UN no: - NOT CLASSIFIED
15. REGULATORY INFORMATION

Hazard symbols: 
No significant hazard.

Safety phrases: 

S2: Keep out of the reach of children.




S26: In case of contact with eyes, rinse immediately with plenty of water and 



seek medical advice.
Air emission directive:
Amount m%: 2

Note: The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions which complete these regulations. Refer to all applicable national, international and local regulations or provisions.

16. OTHER INFORMATION
Other information:
This safety data sheet is prepared in accordance with Regulation (EC)




No 1907/2006.

Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.
