CLEANWELL SAFETY DATA SHEET

OVEN CLEANER


Issued: 01/08/2008







Revision No: 3
1. IDENTIFICATION OF THE  PREPARATION AND SUPPLIER
Product name: 

OVEN CLEANER
Product code: 

304

Use / description of product: 304: An odourless industrial alkali cleaning gel that removes carbonised 



and caramelized oils, proteins and fats from ovens.

Company name: 
CLEANWELL

              

Smithstown Ind. Estate

                              
Shannon, Co. Clare

                              
Telephone             061 360191

                              
Fax:                        061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
SODIUM HYDROXIDE 10-30%

EINECS: 215-185-5 CAS: 1310-73-2

[C] R35
3. HAZARDS IDENTIFICATION

Main hazards: 

Causes severe burns.

4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 
Severe burns may occur. Progressive ulceration will occur if treatment is not immediate.

Eye contact: 
May cause permanent damage. Corneal burns may occur.
Ingestion: 
Nausea and stomach pain may occur. Corrosive burns may appear around the lips.
Inhalation: 
There may be a feeling of tightness in the chest with shortness of breath.

4. FIRST AID MEASURES (ACTION)

Skin contact: 

Drench the affected skin with running water for 10 minutes or longer if 




substance is still on skin. Remove all contaminated clothes and footwear 



immediately unless stuck to skin.

Eye contact: 

Bathe the eye with running water for 15 minutes. Consult a doctor.

Ingestion: 

Do not induce vomiting. Wash out mouth with water. If conscious, give half a 



litre of water to drink immediately. Transfer to hospital as soon as possible.

Inhalation: 

Remove casualty from exposure ensuring one's own safety whilst doing so. 



Transfer to hospital as soon as possible.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Water. Water spray. Carbon dioxide. Alcohol or polymer foam. Dry chemical 



powder.

Exposure hazards: 
Corrosive.

Protection of fire-fighters: Wear protective clothing to prevent contact with skin and eyes.

6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Do not attempt to take action without suitable protective clothing - see section 8 



of SDS. Turn leaking containers leak-side up to prevent the escape of liquid.

Environmental precautions: Do not discharge into drains or rivers.

Clean-up procedures: 
Refer to section 13 of SDS for suitable method of disposal.

7. HANDLING AND STORAGE

Handling requirements: 
Avoid the formation or spread of mists in the air.

Storage conditions: 
Keep container tightly closed.

Suitable packaging: 
Stainless steel. Polyethylene.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Hazardous ingredients: 
SODIUM HYDROXIDE



WEL (15 min STEL): 2 mg/m3

Hand protection: 
Impermeable gloves. Gloves (alkali-resistant). PVC gloves. Viton gloves.

Eye protection: 

Tightly fitting safety goggles. Ensure eye bath is to hand.

Skin protection: 

Alkali resistant protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid

Colour: 


Off-white

Odour: 


Barely perceptible odour

Oxidising: 

Non-oxidising (by EC criteria)

Solubility in water: 
Soluble

Viscosity: 

Highly viscous

Relative density: 
1.178

PH: 


13.5
VOC g/l:

0
10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Conditions to avoid: 
Heat.

Materials to avoid: 
Aluminium. Magnesium. Zinc.

Haz. decomp. products: 
In combustion emits toxic fumes of carbon dioxide. In combustion emits toxic 



fumes of nitrogen oxides. 

11. TOXICOLOGICAL INFORMATION

Hazardous ingredients: 
SODIUM HYDROXIDE




IPR MUS LD50 40 mg/kg




ORL RBT LDLO 500 mg/kg

Routes of exposure: 
Refer to section 4 of SDS for routes of exposure and corresponding symptoms.

12. ECOLOGICAL INFORMATION

Mobility: 

Soluble in water.

Persistence and degradability: Biodegradable.

Bioaccumulative potential: No bioaccumulation potential.

Other adverse effects: 
Negligible ecotoxicity.

13. DISPOSAL CONSIDERATIONS

Disposal of packaging: 
Dispose of as normal industrial waste.

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.

14. TRANSPORT INFORMATION

ADR / RID

UN no: 

1760 


ADR Class: 

8



Packing group: 
II 


Classification code: 
C9



Shipping name: 
CORROSIVE LIQUID, N.O.S. (SODIUM HYDROXIDE)



Labeling: 
8 


Hazard ID no: 

80





    [image: image1.emf]
IMDG / IMO

UN no: 

1760


Class: 


8




Packing group: 
II 


EmS: 


F-A,S-B




Marine pollutant: 


Labeling: 

8

IATA / ICAO

UN no: 

1760


Class: 


8




Packing group: 
II 


Labelling: 

8




Packing instructions: 808(P&CA); 812(CAO)

15. REGULATORY INFORMATION

Hazard symbols: 
Corrosive.



[image: image2.emf]
Risk phrases: 

R35: Causes severe burns.

Safety phrases: 

S26: In case of contact with eyes, rinse immediately with plenty of water and 



seek medical advice.




S36/37/39: Wear suitable protective clothing, gloves and eye / face protection.




S45: In case of accident or if you feel unwell, seek medical advice immediately 



(show the label where possible).




S23: Do not breathe spray.




S28: After contact with skin, wash immediately with plenty of water.




S1/2: Keep locked up and out of the reach of children.

Seveso II guideline: 
No

Water hazard class: 
1 Classification by VwVwS
Air emission directive: 
Amount m%: 0

Note: The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions which complete these regulations. Refer to all applicable national, international and local regulations or provisions.

16. OTHER INFORMATION

Other information: 
12 X 1 LTR; NOT SUBJECT TO ADR

Risk phrases used in s.3: 
R35: Causes severe burns.

Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.

