CLEANWELL SAFETY DATA SHEET

SOLVENT DEGREASER
Issued: 01/01/2010







Revision No: 2
1. IDENTIFICATION OF THE PREPARATION AND SUPPLIER
Product name: 

SOLVENT DEGREASER

Product code: 

710

Use / description of product: 710: A powerfull water rinse able solvent degreaser for fast removal of grease, oil, wax, and bitumen.

Company name: 
CLEANWELL



Smithstown Ind. Estate,


                             Shannon, Co. Clare.

              

               Telephone:            061 360191

                             
Fax:                        061 363762

2. COMPOSITION / INFORMATION ON INGREDIENTS

Hazardous ingredients: 
DODECYLBENZENE SUPHONATE ISOPROPYLAMINE SALT 1-10%



CAS: 26264-05-1
DIBASIC ESTER 1-10% 
CAS: 1119-40-0

[Xi] R36/38

• PETROLEUM DISTILLATES 70-90%

EINECS: 265-149-8 CAS: 64742-47-8
[Xn] R65;   [-] R66
3. HAZARDS IDENTIFICATION

Main hazards: 
Harmful: may cause lung damage if swallowed. Repeated exposure may cause skin dryness or cracking.

4. FIRST AID MEASURES (SYMPTOMS)

Skin contact: 

There may be mild irritation at the site of contact.

Eye contact: 

There may be irritation and redness.

Ingestion: 

There may be soreness and redness of the mouth and throat.

Inhalation: 

There may be irritation of the throat with a feeling of tightness in the chest.

4. FIRST AID MEASURES (ACTION)

Skin contact: 

Wash immediately with plenty of soap and water.

Eye contact: 

Bathe the eye with running water for 15 minutes. Transfer to hospital for 



specialist examination.

Ingestion: 

Do not induce vomiting. Wash out mouth with water. If conscious, give half a 



litre of water to drink immediately. If unconscious and breathing is OK, place in 



the recovery position. Transfer to hospital as soon as possible.

Inhalation: 

Remove casualty from exposure ensuring one's own safety whilst doing so. If 



conscious, ensure the casualty sits or lies down. If unconscious and breathing is 



OK, place in the recovery position. Transfer to hospital as soon as possible.

5. FIRE-FIGHTING MEASURES

Extinguishing media: 
Dry chemical powder. Carbon dioxide
Exposure hazards: 
In combustion emits toxic fumes of carbon dioxide / carbon monoxide.

Protection of fire-fighters: Wear self-contained breathing apparatus.

6. ACCIDENTAL RELEASE MEASURES

Personal precautions: 
Refer to section 8 of SDS for personal protection details.

Environmental precautions: Do not discharge into drains or rivers.

Clean-up procedures: 
Absorb into dry earth or sand.

CLEANWELL SAFETY DATA SHEET

SOLVENT DEGREASER

7. HANDLING AND STORAGE

Handling requirements: 
Ensure there is sufficient ventilation of the area. Do not handle in a confined 



space.

Storage conditions: 
Store in cool, well ventilated area.

Suitable packaging: 
Aluminium containers. Stainless steel. Teflon. Glass.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION
Hazardous ingredients:
PETROLEUM DISTILLATE




WEL (8 hr TWA): 1000mg/m3

Engineering measures: 
Ensure there is sufficient ventilation of the area.

Respiratory protection: 
Gas/vapour filter, type A: organic vapours (EN141).

Hand protection: 
Gloves (solvent-resistant).

Eye protection: 

Safety glasses with side-shields.

Skin protection: 

Solvent resistant protective clothing.

9. PHYSICAL AND CHEMICAL PROPERTIES

State: 


Liquid

Colour: 


Colourless

Odour: 


Characteristic odour

Evaporation rate: 
Moderate

Oxidising: 

Non-oxidising (by EC criteria)

Solubility in water: 
Miscible

Also soluble in: 

Most organic solvents.

Viscosity: 

Non-viscous
Melting point/range oC:
<0
Flash point°C: 

62
Relative density:

0.825

pH:


Non aqueous
VOC g/l: 

790

10. STABILITY AND REACTIVITY

Stability: 

Stable under normal conditions.

Conditions to avoid: 
Flames.

Materials to avoid: 
Oxidising agents.

Haz. decomp. products: 
In combustion emits toxic fumes of carbon dioxide / carbon monoxide.

11. TOXICOLOGICAL INFORMATION

Hazardous ingredients: 
DODECYLBENZENE SUPHONATE ISOPROPYLAMINE SALT




ORL RAT LD50 >2000 mg/kg

DIBASIC ESTER

IHL RAT LC50 11 mg/l

ORL RAT LD50 8191 mg/kg

SKN RAT LD50 2250 mg/kg

12. ECOLOGICAL INFORMATION
Hazardous ingredients:
DODECYLBENZENE SUPHONATE ISOPROPYLAMINE SALT




ALGAE 48H EC50 10 – 300mg/l




DAPHNIA 96H LC50 15mg/l




FISH 96H LC50 1 – 5 mg/l

Mobility: 

Volatile.
CLEANWELL SAFETY DATA SHEET

SOLVENT DEGREASER
Persistence and degradability:
Biodegradable. The surfactants contained in this preparation comply 





with the biodegradability criteria as laid down in regulation (EC) 





No. 648/2004 on detergents.
13. DISPOSAL CONSIDERATIONS

Disposal of packaging: 
Dispose of in a regulated landfill site or other method for hazardous or toxic 



wastes.

NB: The user's attention is drawn to the possible existence of regional or national regulations regarding disposal.

14. TRANSPORT INFORMATION

ADR / RID

UN no: - NOT CLASSIFIED
IMDG / IMO

UN no: - NOT CLASSIFIED
IATA / ICAO

UN no: - NOT CLASSIFIED
15. REGULATORY INFORMATION

Hazard symbols: 
Harmful.
[image: image1.emf]
Risk phrases:

R65: Harmful: may cause lung damage if swallowed.




R66: Repeated exposure may cause skin dryness or cracking.

Safety phrases: 
S62: If swallowed, do not induce vomiting: seek medical advice immediately and show this container or label.
`
S2: Keep out of the reach of children.

Haz. Ingredients (label):
PETROLEUM DISTILLATE
Seveso II guideline: 
Yes

Water hazard class: 
2 Classification by VwVwS

Air emission directive: 
Class: III Item no: 5.2.5 Amount m%: 79
Note: The regulatory information given above only indicates the principal regulations specifically applicable to the product described in the safety data sheet. The user's attention is drawn to the possible existence of additional provisions which complete these regulations. Refer to all applicable national, international and local regulations or provisions.

16. OTHER INFORMATION
Other information:
This safety data sheet is prepared in accordance with Regulation (EC)




No. 1907/2006.
Risk phrases used in s.2: 
R36/38: Irritating to eyes and skin.

R65: Harmful: may cause lung damage if swallowed.

R41: Risk of serious damage to eyes.

R22: Harmful if swallowed

R38: Irritating to skin

R66: Repeated exposure may cause skin dryness and cracking

.

Legal disclaimer: The above information is believed to be correct but does not purport to be all inclusive and shall be used only as a guide. This company shall not be held liable for any damage resulting from handling or from contact with the above product.

